# ACME Water Requirements Specification

Shamal Faily

15-12-22 10:00:00

# Notation

## Risk Analysis Model

## Goal, Obstacle, and Responsibility Model

## Task Model

## Assumption Persona Argumentation Model

# Project Purpose

## Project Background

The security of the environment around Control Systems, operational personnel, sites, assets, activities, information, technological resources, and services has taken on increased importance at ACME Water for the following reasons:

* The disappearing network boundary: a highly mobile workforce works on assets and systems deployed over a diverse and challenging geographic terrain. Information Technology enables this but, in doing so, needs to provide controlled access to core applications, allowing multiple protocols access through the perimeter, reducing perimeter controls, allowing partners to deliver contractual obligations on time and to cost.

* The business climate is highly regulated, and delivery of Wastewater and Cleanwater services are critical to our customers. It is imperative ACME employees are made aware of the importance of protecting our assets, information, and reputation to meet this end.

* Law requires companies to institute reasonable, effective and consistent controls designed to prevent the disclosure and falsification of information, the safety of personnel, sites, and the protection of technological resources.

* Rapid changes face ACME in the use and dependence of computers and network technologies. It is necessary and reasonable to expect everyone to apply the proper methods of handling and safeguarding information, and managing computer and network resources.

## Project Goals

To provide a secure operating environment for SCADA, Telemetry and Control Systems associated with assets owned and operated by ACME

# Project Scope

## Overview

All ICT infrastructure in support of Enterprise SCADA, Telemetry and Control Systems indicated in the rich picture.

## Scope of work

The following rich picture illustrates the scope of this document.

# Environments

## Overview

This paragraph describes the environments within which the planned system will operate in. Properties associated with artifacts such as goals, assets, threats, and vulnerabilties may vary based on these environments, although the system needs to be designed to work in all possible environments.

## Day

### Description

Day-time plant operations

### Short Code

DAY

### Asset Values

| Value | Description |
| --- | --- |
| None | No impact |
| Low | Helps sustain day-to-day operations. |
| Medium | Important for daytime operations. |
| High | Critical to day-to-day operations. |

## Night

### Description

Night-time plant operations

### Short Code

NIGHT

### Asset Values

| Value | Description |
| --- | --- |
| None | No impact |
| Low | Helps sustain night-time operations. |
| Medium | Important for night-time operations. |
| High | Critical to night-time operations. |

# Personas

## Overview

This project needs to be situated for the personas below and the work they carry out. Primary personas should be the main focus of analysis, but it is important not to forget the role other types of persona might play.

## Roles

Personas may fulfil one or more of the below roles. However, roles may also be fulfilled by potential attackers.

Roles

| Name | Description |
| --- | --- |
| Cracker | Cracker |
| Facilities Management | ACME facilities management contractor |
| ICT Partner | ICT partner contractually responsible for delivering ICT services to ACME. |
| Information Security Manager | ACME Information Security Manager |
| Instrument Technician | Instrument Technicians are responsible for maintaining operational SCADA, Telemetry, and Control System software. |
| Integrator | Work with process partners to implement the control philosophy associated with a capital project |
| Machine | System hardware |
| Petty Criminal | Engages in acts of petty crime |
| Plant Operator | Plant operators monitor the water treatment process at a clean or waste water treatment plant. |
| Service Desk | Service Desk staff acting as first-line support for any ICT problems. |
| Vendor | Vendor of SCADA, Telemetry, and Control System software |

## Personas

### Barry

#### Type

Secondary

#### Activities

Much of Barry's work involves equipment modification and instrument calibration; this work arises from requests from Process, or activities as part of on-going projects to improve plant efficiency. As part of these changes, Barry may need to modify alarms on outstations, and make minor changes to PLCs and HMIs.

Barry is often called out to troubleshoot problems at sites and kiosks within his area of operations in Dorset, which can also involve software changes ranging from simply downloading software to a device to clean up its memory, through to re-generating the software from configuration sheets if no software backup is available.

#### Attitudes

Barry has learned the hard way that you only start learning when things go wrong. As such, Barry tries to keep up-to-speed with planned SCADA, PLC, and outstation changes as much as possible; he knows that avenues of support may not be easily available in 5-10 years when a piece of equipment is modified, and the associated software needs to change.

If he finds out that some plant modifications are planned in his area, Barry will try to get hold of the relevant work manual; he knows that otherwise he won't be able to get hold of documentation which might be useful in the future.

Although aware of security, Barry is not exceptionally worried about it. After all, no-one really knows anything about the software he looks after and, in terms of physical security, all the manned site on his patch have security guards.

#### Aptitudes

Barry has been using many of his day-to-day software tools almost as long as the tools have been around. Barry knows quite a bit about telemetry software, so often gets called up by the more junior instrument techs to deal with any telemetry issues. Conversely, Barry is less experienced with some of the SCADAs in use, so is reliant on people working in plants, or other colleagues, for any detailed troubleshooting.

Due to the lack of ACME ICT support for PC hardware at his site, Barry has learned how to backup his site PCs, his personal laptop, and recover from PC hardware failures - a significant problem given the age of some of the PCs in his area.

#### Motivations

Barry gets annoyed when he needs to make a software change at a remote site, and finds that he can't get at the software he needs from a PLC or outstation, can't trust the on-site backup left behind or the software on his laptop.

#### Skills

Barry doesn't have any specific experience of software repositories or version control. He does, however, have lots of experience archiving and restoring PLC and telemetry software.

#### Intrinsic

None

#### Contextual

None

#### Day Roles

Barry role attributes

| Roles | Direct/Indirect |
| --- | --- |
| Instrument Technician | False |

#### Day Security Issues

On any given day, Barry makes several modifications and several different locations. Therefore, it is important that the interface for working with the repository is built around Barry, rather than the other way around.

#### Night Roles

Barry role attributes

| Roles | Direct/Indirect |
| --- | --- |
| Instrument Technician | False |

#### Night Security Issues

On any given day, Barry makes several modifications and several different locations. Therefore, it is important that the interface for working with the repository is built around Barry, rather than the other way around.

### Rick

#### Type

Primary

#### Activities

Rick is a clean-water plant operator, responsible for ensuring that water processed through the plant is in line with specified set-points. Rick's area of responsibility extend outside of the confines of the plant to include checking on pumping stations, which feed water into the plant's nearby reservoir.

During his working shifts (he works a 5 + 1 continental shift), Rick frequently checks readings on his SCADA workstation, takes water samples at various locations around the plant, and responds to alarms raised by the SCADA system; if necessary this involves investigating problems, and -- if no remedial tasks are possible -- calling someone out to investigate further.

At the start of every shift, Rick checks AJS to see what work he is scheduled to carry out. He usually updates AJS based on the work he carried out before the end of his shift. Rick also maintains a paper work diary, which is updated after carrying out specific work or responding to problems. This, together with the status white board, is also useful for explaining any on-going issues when handing over to the next shift.

#### Attitudes

Rick isn't particularly concerned about people trying to hack into the SCADA system he uses. "The only way the SCADA will get infected", Rick says, "is by an instrument tech plugging a virus infected laptop into it".

Rick is aware of Information Security communiques on InfoPortal, but doesn't see how this is relevant to him. "We don't have laptops", says Rick, "and we don't leave site, so we don't leave anything in cars". Rick is, however, aware of Information Security controls. Rick knows that logout policies keep intruders from getting access to his workstation (although he thinks the logout period is too short given his environment), and he knows he can only access certain pumping sites via the telemetry system.

Although Rick can call an instrument tech for many problems, experience has taught him not to expect much from the ACME ICT support during a night shift. "What's the point of calling up to say you can't login at night if all you get is a reference number", Rick complains, "that won't help me start my pumps now!" Although Rick knows he can call out an instrument tech to help, he is also conscious that they are out most evenings, and might not appreciate a call-out for a corporate ICT problem.

#### Aptitudes

Rick has been a plant operator for over 10 years. Although the exact procedures need to run his plant are documented in a works manual, he doesn't need to refer to it. "I haven't read the works manual in years", Rick says.

This experience does not stay in Rick's head. When problems have occurred, he raises these during team meetings, and any lessons learned sometimes find their way into the SCADA system as new functionality. Rick is aware of the new EnterpriseSCADA developments and possible improvements to the HMI and start-up procedure. Although not fully in the loop about what is going on, he hopes that, like the current system, there will be productivity improvements without an adverse impact.

Rick uses the SCADA's trend monitoring facilities as a indication of possible problems. "If the trends freeze or flat-line", Rick says, "I need to go out and take a look at what's wrong". Because problems occur daily, and he is often on his own, Rick can still control the plant using panel PCs at remote locations around the plant.

#### Motivations

Because the water quality can change over a matter of minutes, Rick's ethos is that you need to be ready for anything. Because of this, checking certain parameters and pieces of kit is an important discipline, as is logging important events in the works diary and on the status board; these aid communication to other shift members, and the eventual piecing together of the elements of an event.

Although information security doesn't phase Rick too much, personal security does. Potentially facing off a scrap metal thief is a big worry for Rick. "The police don't respond to intruder alarms at a nearby pumping station any more due to false alarms", says Rick. "Because of this, we've been told not to go out to these places on our own. We have a lone-worker system when people call us when we get to a particular station, but what happens if we get problems on the way?"

#### Skills

Rick has a basic operating knowledge of Windows; enough to carry out his day-to-day routines, check his email, and navigate the main sections of InfoPortal.

Rick isn't a power-user; he acknowledges that younger "whizz kids" are much more adept at using PCs than he is. Consequently, he only uses a sub-set of the desktop icons currently on his desktop. "I don't know what these other applications do", Rick says,"I've tried clicking on one and it just gives me a Data Protection Warning. I guess this means I'm not authorised to use it."

#### Intrinsic

None

#### Contextual

None

#### Day Roles

Rick role attributes

| Roles | Direct/Indirect |
| --- | --- |
| Plant Operator | False |

#### Day Security Issues

Rick carries out certain jobs during the day. These include ordering, and taking delivery of chemicals

#### Night Roles

Rick role attributes

| Roles | Direct/Indirect |
| --- | --- |
| Plant Operator | False |

#### Night Security Issues

Certain plant operations are carried out at night because the cost of electricity is cheaper. However, much of Rick's night shifts are spent in-doors, and the control room doors are locked to keep out possible intruders.

#### Rick Argumentation Model

##### Activities

##### Attitudes

##### Aptitudes

##### Motivations

#### Document References

Rick Document References

| Reference | Document | Excerpt |
| --- | --- | --- |
| Alarms designed around past experience | Site knowledge GT concept | Software people asked to set up alarms based on past problems forgetting about alarms. |
| Anything could happen | Process impact GT concept | Anything could happen: pumps could fail; water quality can change within 10 minutes |
| Bespoke systems harder to understand and support. | Limited support knowledge GT concept | Bespoke systems have problems that it takes a while to understand how it has been programmed. |
| Callouts occur for anything other than a basic trip. | Alarm handling GT concept | Basic trips can be manually reset but anything more complicated warrants a call-out. |
| Carries out tasks and records set-point readings | Work routine GT concept | Looks at tasks needed to be achieved in a day and set-points to be collected and recorded. |
| Checking certain things is a discipline | Site knowledge GT concept | Checking certain things e.g. coagulant levels is a discipline. |
| Checks take place at certain times of the day | Trend monitoring GT concept | Checks are monitored throughout the works at certain times during the day. |
| Copper theft | Site threats GT concept | Sites have been broken into to steal earth connections and put through mains to get at copper. |
| Corporate wifi insecurity | InfoSec indifference GT concept | The corporate wifi is not perceived to be secure as the access code is written down on a wall. |
| Drifting trends indicate problems | Trend monitoring GT concept | Trends drifting off are an indication that something has gone out of sync. |
| Environmental Agency carry out river monitoring | External agencies GT concept | The Environmental Agency carry out their own river monitoring. |
| Everything is logged | Business compliance GT concept | Everything that happens is logged. |
| Experience contributes to asset management decisions. | Site knowledge GT concept | Experience about what goes wrong contributes to asset purchase decisions via AJS. |
| Extreme scenarios | Unlikely threats GT concept | Scenarios of corrupt or missing files taking out a workstation are a bit extreme. |
| Failures often occur when seasons change. | Hazards GT concept | Failures often occur when seasons change. |
| Familiarity with colour scheme standards. | Site knowledge GT concept | People coming in from different plant are familiar with colour schemes. |
| Few SCADA to EnterpriseSCADA process changes | Process impact GT concept | Few process changes expected when old SCADA systems replaced with EnterpriseSCADA. |
| Hacker aware | InfoSec communication GT concept | Has read warning about people trying to hack machine on InfoPortal. |
| Hacking indifference | InfoSec indifference GT concept | Not worried about people trying to hack the SCADA systems. |
| HMI usability improvements | Process Innovation GT concept | The new SCADA system is an improvement because there are only slight differences to the MMI. |
| Infection only from engineers | Island mindset GT concept | A system only gets infected if an engineer connects their own infected laptop into it. |
| InfoSec communiques irrelevant | InfoSec indifference GT concept | Messages about leaving things and laptops in a car are not a problem; [we] do not go off site. |
| Lack of night ICT support | ICT non-support GT concept | Why have night support if all you get is a logged reference number; you cannot hand the problem over to someone else. |
| Line manager site authorisation | Authorisation restriction GT concept | Can only access sites they have been authorised to; permission for authorisation changes need to be sought from the line manager. |
| Lone worker vulnerability | Personal Security GT concept | Lone Working system is based on calling up someone when they get to a station; it does not help if you get problems on the way. |
| Many desktop short-cuts do not get used. | Unexplained feature GT concept | Most short-cuts and applications on the desktop have been around a while and do not get used. |
| Massive and uncontrollable water input | Hazards GT concept | It is not like a factory: it is a massive system with tonnes of debris that hits you all in one go. |
| Migration pending resources | Interim Systems GT concept | Old PC applications will be migrated to SCADA when time and money is available. |
| MS activities during start of day shift. | Business compliance GT concept | Management System activities are carried out during the first hours of a day shift. |
| MS workaround for intruder alarms. | Business compliance GT concept | Lack of police response to intruder alarms led to two-man requirement for alarm response. |
| Night shift tasks differ slightly from day. | Work routine GT concept | Night shift tasks may be similar but slight different due to SAS sampling |
| No alone out-of-hours visits | Personal Security GT concept | Never go alone when visiting a nearby site out-of-hours |
| Non-authorised applications cannot be run | Unexplained feature GT concept | Not being able to run applications means not authorised to use it. |
| Notes are easier to read and hand-over | Works diary GT concept | Easier to write things on notes. Also easier for person handed over to to just read the logs. |
| Only general work and failures are logged | Works diary GT concept | Supposed to write down everything but not enough pages; any general work carried out is logged - together with failures. |
| Operators know what they need to do. | Work routine GT concept | Operators come in and know what work they need to do. |
| Out-of-spec alarms precipitate TIS call-outs | External agencies GT concept | Out of spec alarms are picked up Telemetry Information System; these lead to a technician call out; independent sampling; log checking. |
| Periodic national security alerts | big security worry GT concept | Regular emails are received about things like the Iraqi Wars and terrorism; it then goes quite for a while until the next event. |
| Personal threats | Personal Security GT concept | Personal experience of being threatened when called out late at night and challenging intruders. |
| Physical and login security | InfoSec indifference GT concept | Once you are through the gate then security is the operator login. |
| Plant can be operated away from the control room if necessary. | One-man site GT concept | Can run things from the other side of the plant; this is necessary when you are on your own. |
| Process problems occur daily. | Process impact GT concept | Process problems occur daily. |
| Processes checked against spec | Scope of responsibility GT concept | Check the process and quality of final effluent meets critical parameters with EA specs. |
| Pumping decisions rain based | Hazards GT concept | Decisions to pump are based on the rain forecast. |
| Quick and automatic startup | Process Innovation GT concept | The workstation has been setup so as soon as Windows starts then so does EnterpriseSCADA (as a service); no user interaction is needed. |
| Readings are taken from SCADA screens. | Hardware checks GT concept | SCADA screens are checked to take readings about chemical dosing; iron levels; pH etc. |
| Records piece things together | Site knowledge GT concept | Records are kept to help piece things together when things go wrong. |
| Role restrictions | Authorisation restriction GT concept | Depending on what you are authorised to do - certain buttons for things like starting and stopping pumps will be greyed out. |
| Room for improvement | Process Innovation GT concept | There is always scope to improve and automate as we go along. |
| Shift based logouts | Logout policy GT concept | People want a shift based auto-logout feature. |
| Short auto logout | Logout policy GT concept | Automatic logout occurs after a minute or two of inactivity. |
| Site kept running at optimum efficiency | Scope of responsibility GT concept | Keep site running at optimum efficiency |
| Some plant operations are cheaper at night. | Operational contexts GT concept | Pumps are run from some districts at night because the electricity is cheaper. |
| Specific jobs are day shift related | Work routine GT concept | Specific jobs are assigned to the day shift e.g. ordering chemicals |
| Stand-alone SCADA | Island mindset GT concept | Systems are stand-alone because they are not connected a network. |
| Standards evolved over years | Local standards GT concept | Standards have been setup over the years. |
| Status board communicates problems | Site knowledge GT concept | Status board is used to communicate problems with bits of kit. |
| Tasks entered into AJS are captured by WMS | Corporate interface GT concept | Tasks are entered into Asset Job System are captured by Work Management System. |
| The works diary is a controlled document. | Business compliance GT concept | The works diary is a controlled document. |
| Thieves steal anything | Site threats GT concept | Thieves will steal anything. |
| Trends identify plant problems | Trend monitoring GT concept | Trends are used to identify plant problems. |
| Trouble-shooting driven by known task problems. | Site knowledge GT concept | Knowledge about coagulant problems leads to particular tasks being carried out e.g. flushing lines; to get everything back to normal. |
| Uncertainty about EnterpriseSCADA ability to support general operations. | Limited support knowledge GT concept | Uncertainty about EnterpriseSCADA capabilities towards supporting general operations. |
| Untimely logout annoyance | ICT non-support GT concept | It is annoying to nip downstairs and have to log back in when you come back. |
| Vandalism assessed and mitigated | Site threats GT concept | Vandalism has already been assessed and mitigated. |
| Water entering plant checked against spec | Scope of responsibility GT concept | Make sure water is in-spec coming into the plant. |
| Water is sampled throughout the works | Sample collection GT concept | All aspects of the water down in the works is sampled. |
| Water scheme is geographically big | Water GT concept | The scheme covers where water is sourced 40 miles away; it has a long way to get to the works. |
| Works manual is internalised | Site knowledge GT concept | Works manual has been internalised. |

#### External Documents

Rick External Documents

| Document | Version | Authors | Date | Description |
| --- | --- | --- | --- | --- |
| Alarm handling GT concept | 1 | Shamal Faily | August 2010 | Alarm handling GT concept |
| Authorisation restriction GT concept | 1 | Shamal Faily | August 2010 | Authorisation restriction GT concept |
| big security worry GT concept | 1 | Shamal Faily | August 2010 | big security worry GT concept |
| Business compliance GT concept | 1 | Shamal Faily | August 2010 | Business compliance GT concept |
| Corporate interface GT concept | 1 | Shamal Faily | August 2010 | Corporate interface GT concept |
| External agencies GT concept | 1 | Shamal Faily | August 2010 | External agencies GT concept |
| Hardware checks GT concept | 1 | Shamal Faily | August 2010 | Hardware checks GT concept |
| Hazards GT concept | 1 | Shamal Faily | August 2010 | Hazards GT concept |
| ICT non-support GT concept | 1 | Shamal Faily | August 2010 | ICT non-support GT concept |
| InfoSec communication GT concept | 1 | Shamal Faily | August 2010 | InfoSec communication GT concept |
| InfoSec indifference GT concept | 1 | Shamal Faily | August 2010 | InfoSec indifference GT concept |
| Interim Systems GT concept | 1 | Shamal Faily | August 2010 | Interim Systems GT concept |
| Island mindset GT concept | 1 | Shamal Faily | August 2010 | Island mindset GT concept |
| Limited support knowledge GT concept | 1 | Shamal Faily | August 2010 | Limited support knowledge GT concept |
| Local standards GT concept | 1 | Shamal Faily | August 2010 | Local standards GT concept |
| Logout policy GT concept | 1 | Shamal Faily | August 2010 | Logout policy GT concept |
| One-man site GT concept | 1 | Shamal Faily | August 2010 | One-man site GT concept |
| Operational contexts GT concept | 1 | Shamal Faily | August 2010 | Operational contexts GT concept |
| Personal Security GT concept | 1 | Shamal Faily | August 2010 | Personal Security GT concept |
| Process impact GT concept | 1 | Shamal Faily | August 2010 | Process impact GT concept |
| Process Innovation GT concept | 1 | Shamal Faily | August 2010 | Process Innovation GT concept |
| Sample collection GT concept | 1 | Shamal Faily | August 2010 | Sample collection GT concept |
| Scope of responsibility GT concept | 1 | Shamal Faily | August 2010 | Scope of responsibility GT concept |
| Site knowledge GT concept | 1 | Shamal Faily | August 2010 | Site knowledge GT concept |
| Site threats GT concept | 1 | Shamal Faily | August 2010 | Site threats GT concept |
| Trend monitoring GT concept | 1 | Shamal Faily | August 2010 | Trend monitoring GT concept |
| Unexplained feature GT concept | 1 | Shamal Faily | August 2010 | Unexplained feature GT concept |
| Unlikely threats GT concept | 1 | Shamal Faily | August 2010 | Unlikely threats GT concept |
| Water GT concept | 1 | Shamal Faily | August 2010 | Water GT concept |
| Work routine GT concept | 1 | Shamal Faily | August 2010 | Work routine GT concept |
| Works diary GT concept | 1 | Shamal Faily | August 2010 | Works diary GT concept |

# Mandated Constraints

Constraints

| Name | Type | Description |
| --- | --- | --- |
| PC infrastructure | Invariant | Works diary GT concept |

# Naming Conventions

Naming Conventions

| Name | Definition |
| --- | --- |
| ACME | ACME Water |
| AJS | Asset Job System |
| DMZ | Demilitarised Zone |
| EnterpriseSCADA | Enterprise SCADA system currently being deployed to support the Advanced Digester project. |
| ICT | Information and Communications Technology |
| MCC | Motor Control Center |
| RFC | Request for change |
| SCADA | Supervisor Control and Data Acquisition |
| STCS | Scada, Telemetry, and Control Systems |
| TMS | A family of products for providing continuous monitoring of the asset estate. |
| USB | Universal Serial Bus |
| WMS | Work Management System |

# Assets

## Overview

This chapter describes the most important assets in, or associated with, the planned system.

## Asset Types

Asset Types

| Type | Description |
| --- | --- |
| Information | Documented (paper or electronic) data or Intellectual Property used to meet the mission of an organisation. |
| Systems | Information Systems that process and store information (systems being a combination of information, software, and hardware assets and any host, client, or server being considered a system). |
| Software | Software application and services -- such as operating systems, database applications, networking software, office applications, custom applications, etc. -- that process, store, or transmit information. |
| Hardware | Information Technology physical devices -- such as workstations, servers, etc -- that normally focus solely on the replacement costs for physical devices. |
| People | The people in an organisation who possess unique skills, knowledge, and experience that are difficult to replace. |
| Systems - General | Organisational, Groups, and Social Systems, of which may have, use, manage, and operate Information Systems that process, store, or transmit information. |
| System of Systems | The combination of Independent Systems (collaborating to achieve a new combined purpose and goal). |

## Asset Properties

Asset Properties

## Models

### Day

### Night

## Corporate Network

Corporate Network attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | ACME Corporate network. |
| Significance | Compromising the network may compromise the data passing through it. |

Corporate Network environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Confidentiality : Medium (Network traffic contains commercially confidential data.) Integrity : Medium () Availability : Medium () Accountability : Medium () |
| Night |  |

## Diary Entry

Diary Entry attributes

| Attribute | Description |
| --- | --- |
| Type | Information |
| Description | Entry logged by plant operator to works diary |
| Significance | Audit trail for how anomalous operations handled. |

Diary Entry environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Accountability : Medium (Important to know who updated diary in the event of an incident.) |
| Night | Accountability : Medium (Important to know who updated diary in the event of an incident.) |

## Enterprise SCADA Network

Enterprise SCADA Network attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Enterprise Control Network, connecting PLCs, EnterpriseSCADA servers, and EnterpriseSCADA clients |
| Significance | Compromising this network potentially compromises any assets connected to it. |

Enterprise SCADA Network environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Confidentiality : High (Network traffic reveals information about the configuration of connected devices, and sensitive information that may be used to control them..) Integrity : High (Tampering with network traffic affectss ACME's ability to manage the plant infrastructure.) Availability : Medium (Inaccessibility makes maintainability of connected devices difficult.) Accountability : High (Changes to the network and network infrastructure need to be logged to assist troubleshooting activities.) |
| Night |  |

## EnterpriseSCADA Server

EnterpriseSCADA Server attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | EnterpriseSCADA works server. Connects to PLCs and routers |
| Significance | Compromising the EnterpriseSCADA server also compromising any connected clients and PLCs |

EnterpriseSCADA Server environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : High (Tampering could affect all connected PLCs and routers.) Availability : Medium (Inaccessibility impacts the maintainability of the connected SCADA infrastructure.) Accountability : High (Need to know who has maintained this system.) |
| Night |  |

## Firewall

Firewall attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Firewalls connecting the local works network with the corporate network and the enterprise SCADA network. |
| Significance | Compromising the firewalls compromises systems both within the works, and any external systems reliant on works data. |

Firewall environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Medium (Modifications to the firewall could expose the works network to attack.) Availability : High (Compromising the firewall could lead to an unacceptable degrading of performance of application on the works network.) Accountability : Medium (Useful to know who has maintained this system.) |
| Night |  |

## ICT Application

ICT Application attributes

| Attribute | Description |
| --- | --- |
| Type | Software |
| Description | Standard build ICT application |
| Significance | A compromised ICT application could affect other applications running on the same machine. |

ICT Application environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Low (Tampering might make affect general machine performance, affecting access to more critical applications.) Availability : Low (Productivity application unavailability might impact certain tasks.) Accountability : Low (ICT applications should be maintained only by authorised ICT staff.) |
| Night |  |

## ICT cabinet

ICT cabinet attributes

| Attribute | Description |
| --- | --- |
| Type | Hardware |
| Description | Racks containing server and network equipment |
| Significance | Damaging the cabinet compromises all the assets on it. |

ICT cabinet environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Medium (Tampering could lead to physical damage to systems on the rack.) Availability : High (Physical damage to the cabinet affects all systems on the cabinet.) |
| Night |  |

## ICT credentials

ICT credentials attributes

| Attribute | Description |
| --- | --- |
| Type | Information |
| Description | ACME ICT login credentials. Based on WMS pay number. |
| Significance | Obtaining some or all of ICT credentials may allow an unauthorised person to logically impersonate an authorised principal |

ICT credentials environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Confidentiality : Medium (Unauthorised access to credentials may lead to access to sensitive corporate data.) Availability : Low (Loss of credentials hampers tasks requiring authentication.) Accountability : Low (Credentials should be issued only to authorised ACME staff) |
| Night | Confidentiality : Medium (Unauthorised access to credentials may lead to access to sensitive corporate data.) Availability : Medium (Loss of credentials may significantly hamper tasks requiring authentication, due to lack of ICT helpdesk support.) |

## ICT PC

ICT PC attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | ICT PC with TMS. Also used to access InfoPortal and corporate ICT applications, such as Outlook |
| Significance | Facilitates access to corporate ICT and telemetry services to authorised principals. Turns pumps on and off |

ICT PC environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Medium (Tampering with the PC could affect STCS applications running on the ICT network.) Availability : Medium (Unavailability affects tasks relying on productivity applications.) Accountability : Low (PCs should be used only by authorised ACME employees.) |
| Night | Integrity : Medium (Tampering with the PC could affect STCS applications running on the ICT network.) Availability : Medium (Unavailability affects tasks relying on productivity applications.) Accountability : Low (PCs should be used only by authorised ACME employees.) |

## InfoPortal

InfoPortal attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | ACME portal |
| Significance | Front-end portal for accessing other ICT services |

InfoPortal environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Availability : Low (Provides useful information to support day-to-day tasks.) |
| Night |  |

## Intrusion Detection System

Intrusion Detection System attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Intrusion Detection System. Used to detect unusual or unauthorised network activitiy |
| Significance | False positives may render the network usable. Missing rules may cause unauthorised access to go unfettered. |

Intrusion Detection System environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : High (Tampering with rules might lead to excessive false positives or negatives that severely damage network operations.) Accountability : Medium (It is critical that IDS changes are made only by authorised staff.) |
| Night |  |

## Laptop

Laptop attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Instrument Tech laptop. Development tools installed for uploading software to and from outstations and PLCs. |
| Significance | Compromising the laptop could compromise any connected systems. |

Laptop environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : High (Compromised laptop could compromise control systems they connect to.) Availability : High (Unavailable laptops stymies ability to patch PLCs and other equipment.) Accountability : Medium (Laptops should be issued and used only to authorised technicians.) |
| Night | Integrity : High (Compromised laptop could compromise control systems they connect to.) Availability : High (Unavailable laptops stymies ability to patch PLCs and other equipment.) Accountability : Medium (Laptops should be issued and used only to authorised technicians.) |

## Modem

Modem attributes

| Attribute | Description |
| --- | --- |
| Type | Hardware |
| Description | Modem attached to SCADA or ICT workstation. |
| Significance | May give rise to unauthorised outgoing or incoming network connections. |

Modem environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Medium (Tampering may lead to subversive changes.) Availability : High (Several tasks on legacy systems rely on modem communications.) |
| Night |  |

## Plant Staff

Plant Staff attributes

| Attribute | Description |
| --- | --- |
| Type | People |
| Description | Plant staff |
| Significance | Intangible assets |

Plant Staff environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Availability : High (Operations contingent on human staff) |
| Night | Availability : Medium (Operations contingent on human staff) |

## PLC

PLC attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Programmable Logic Controller. |
| Significance | Compromising PLCs can compromise connected assets, and other assets associated with it. |

PLC environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : High (Tampering impacts monitoring of water quality.) Availability : High (Controls machinery) |
| Night | Integrity : High (Tampering impacts monitoring of water quality.) Availability : High (Controls machinery) |

## PLC Event

PLC Event attributes

| Attribute | Description |
| --- | --- |
| Type | Information |
| Description | Event message indicating some PLC anomaly. |
| Significance | Needs to be attended to by plant operators when raised. If not send, received, or inappropriately suppressed, hazards may arise. |

PLC Event environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Medium (Spoofed or tampered PLC events may lead to false or negative positives.) Availability : High (Indicates potential hazards) |
| Night | Integrity : Medium (Spoofed or tampered PLC events may lead to false or negative positives.) Availability : High (Indicates potential hazards) |

## Router

Router attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Routers route and direct network traffic |
| Significance | Modifying a router's configuration enables packer redirection and sniffing. |

Router environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Confidentiality : Low (Configuration information about the routers could facilitate attacks on the network.) Integrity : Medium (Tampering could facilitate packet redirection and sniffing.) Availability : Medium (Unavailable routers could compromise availability of connected devices, which could impact day-to-day operations.) |
| Night |  |

## SCADA Workstation

SCADA Workstation attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | SCADA workstation. Runs SCADA or EnterpriseSCADA client software. |
| Significance | Detects alarms. Monitors trends. Turns kit on and off. |

SCADA Workstation environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : High (Compromising the workstation could compromise any connected systems.) Availability : High (Unavailable workstations could leads to delays monitoring operations, and responding to anomalies.) Accountability : High (Workstations should be used only by authorised plant staff.) |
| Night | Integrity : High (Compromising the workstation could compromise any connected systems.) Availability : High (Unavailable workstations could leads to delays monitoring operations, and responding to anomalies.) Accountability : High (Workstations should be used only by authorised plant staff.) |

## STCS Application

STCS Application attributes

| Attribute | Description |
| --- | --- |
| Type | Software |
| Description | SCADA, Telemetry, Control System application |
| Significance | Water Treatment process could be compromised by corrupted or non-available software. |

STCS Application environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : High (Tampering with STCS settings could hava a catastrophic impact to the plant and broader environment.) Availability : Medium (Awkward for operators if STCS Applications are unavailable.) Accountability : Low (Changes to STCS settings should be tracked.) |
| Night | Integrity : High (Tampering with STCS settings could hava a catastrophic impact to the plant and broader environment.) Availability : Medium (Awkward for operators if STCS Applications are unavailable.) Accountability : Low (Changes to STCS settings should be tracked.) |

## STCS credentials

STCS credentials attributes

| Attribute | Description |
| --- | --- |
| Type | Information |
| Description | Credentials allowing an authorised user to be authenticated with a SCADA, Telemetry, or Control System client |
| Significance | Obtaining STCS credentials may allow an unauthorised person to logically impersonate an authorised principal |

STCS credentials environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Confidentiality : High (Access to the credentials could lead to catastrophic misuse of attached systems.) Accountability : Medium (Credential management should be tracked.) |
| Night |  |

## Suppress Event

Suppress Event attributes

| Attribute | Description |
| --- | --- |
| Type | Information |
| Description | Event message indicating an operator has overriden an alarm |
| Significance | If alarms are inappropriately suppressed, hazards may arise. |

Suppress Event environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Medium (Spoofed or tampered failure events may lead to false or negative positives.) Availability : Medium (Indicates false positives which, if not suppressed, can disrupt operations) |
| Night | Integrity : Medium (Spoofed or tampered failure events may lead to false or negative positives.) Availability : Medium (Indicates false positives which, if not suppressed, can disrupt operations) |

## Telemetry Network

Telemetry Network attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Telemetry Network. Used to passing alarms from outstations and pumps, and sending commands to remote kit via TMS |
| Significance | Compromising the telemetry network risks disrupting alarms sent to the bunker, or commands sent to remote kit. |

Telemetry Network environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Confidentiality : Low (Network traffic may reveal information about the configuration of devices on the network.) Integrity : High (Tampering with network traffic affects ACME's ability to monitor the water infrastructure if data is false or inaccurate.) Availability : High (Inaccessibility affects ACME's ability to monitor the water infrastructure.) Accountability : High (Changes to the network and network infrastructure need to be logged to assist troubleshooting activities.) |
| Night |  |

## Works Diary

Works Diary attributes

| Attribute | Description |
| --- | --- |
| Type | Information |
| Description | Diary of everything notable during each shift. |
| Significance | Audit trail for how anomalous operations handled. |

Works Diary environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Integrity : Medium (Tampering could compromise accountability) Accountability : Medium (Important to know who updated diary in the event of an incident.) |
| Night | Integrity : Medium (Tampering could compromise accountability) Accountability : Medium (Important to know who updated diary in the event of an incident.) |

## Works Network

Works Network attributes

| Attribute | Description |
| --- | --- |
| Type | Systems |
| Description | Works network connecting PLCs, local outstations, and SCADA applications |
| Significance | Compromising or disrupting the communication between devices on the SCADA network compromises the work of the SCADA application. |

Works Network environmental attributes

| Environment | Security Property (Rationale) |
| --- | --- |
| Day | Confidentiality : Medium (Network traffic reveals information about the configuration of devices on the network.) Integrity : High (Tampering with network traffic affects ACME's ability to manage the plant infrastructure.) Accountability : High (Changes to the network and network infrastructure need to be logged to assist troubleshooting activities.) |
| Night | Confidentiality : Medium (Network traffic reveals information about the configuration of devices on the network.) Integrity : High (Tampering with the network traffic affects ACME's ability to manage the plant infrastructure.) Accountability : High (Changes to the network and network infrastructure need to be logged to assist troubleshooting activities.) |

# Tasks

## Overview

This chapter describes the work tasks which the planned system will need to be designed for. These tasks incorporate some level of interactivity with the planned system, and are carried out by the personas. Tasks are also scored based on how usable these are to the personas who carry them out.

## Models

### Day

### Night

## Broken instrument alarm

### Objective

Fix flatlined trend

### Environments

#### Day

##### Dependencies

Rick hears an audible alarm and notices that the trends have flatlined.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Rick | Minutes | Hourly or more | Low | Low |

##### Assets

Assets

Asset

STCS Application

##### Narrative

Rick looks at the SCADA terminal and notices one of the inlet pumps appears to have failed. Rick leaves the control room building, and walks to the building where the offending inlet pump lives. Once he gets there, he doesn't see anything obviously wrong, so he checks the MCC to see if there has been some sort of trip. Again, Rick can't find anything amiss with the electrics, so he concludes there must be some sort of communications problem.

Rick returns back to the control room, and calls up one of the instrument techs located in a building on the other side of the site. After Rick explains the problem and puts the phone down, he stops the alarm and notes his actions so far in the works diary. After the 15 minutes, Rick gets a phone call. As he picks the phone up, he notices that trends have returned to normal again. Barry explains that there was a minor fault due to a spurious PLC setting, but everything is back to normal again. After this call, Rick again notes the event in the works diary.

#### Night

##### Dependencies

Rick hears an audible alarm and notices that the trends have flatlined.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Rick | Hours or longer | Hourly or more | Low | Low |

##### Assets

Assets

Asset

Works Diary

STCS Application

##### Narrative

Rick looks at the SCADA terminal and notices one of the inlet pumps appears to have failed. Because Rick is the only person currently on site, Rick leaves the control room building, locking all the doors behind him, and walks to the building where the offending inlet pump lives. Once he gets there, he doesn't see anything obviously wrong, so he checks the MCC to see if there has been some sort of trip. Again, Rick can't find anything amiss with the electrics, so he concludes there must be some sort of communications problem.

Rick returns back to the control room, and calls up the bunker at Dorchester. He describes the problem, and asks for a stand-by instrument tech to be called out to take a look. After Rick puts the phone down, he stops the alarm and notes his actions so far in the works diary. After an hour, Rick glances up at the CCTV camera and notices a white transit van approaching, after stopping at the gate for a few moments, the electrified gate opens up and the van drives through. This van is driven by Barry, an instrument tech who is a frequent visitor to the site. After a few minutes, Rick hears the buzzer for the main entrance to the building, after hearing Barry's voice, Rick buzzes Barry in. When Barry gets to the control room and exchanges a few pleasantries, Rick explains the problem to Barry, who heads down to the inlet pump building.

Fifteen minutes later, Barry arrives back at the control room and explains there was a minor fault due to a spurious PLC setting, but everything is back to normal again. Rick glances over to the SCADA terminal and notices that the trends are indeed back to normal. After Barry leaves, Rick notes the event in the works diary.

## Modify PLC software

### Objective

Modify the PLC software in order to maintain plant operations.

### Environments

#### Day

##### Dependencies

Barry has looked at the code before hand, so is aware of what amendments need to be made. Barry knows the PLC he needs to modify. A job has been raised on SAP and picked up.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Barry | Hours or longer | Hourly or more | High | High |

##### Assets

Assets

Asset

Laptop

STCS Application

##### Narrative

Before Barry reaches the site, he speaks to Rick to discuss the job requirements. As soon as Barry has this understanding, he downloads the latest changes to his laptop and makes the necessary PLC software modifications.

Barry walks over to the panel which accesses the PLC needing to be modified. Barry then starts up his laptop. Barry checks the version of his software matches that which is on the PLC. This comparison is carried using Step 7. Following this, Step 7 is used to download the software modifications to the PLC.

After downloading, Barry carries out a number of site operation checks to commision the software; this involves taking the piece of equipment out of service in order to make sure the plant operates. These checks involve co-ordination with Rick.

After the site operations check, Barry backs up the software. Finally, Barry speaks to Rick to make sure he is happy with the changes made. Barry then closes the job down using his hand-held.

#### Night

##### Dependencies

Barry has been verbally given information about the job from Rick. Barry makes a decision to download the latest version of the PLC software based on local factors.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Barry | Hours or longer | Hourly or more | High | High |

##### Assets

Assets

Asset

Laptop

STCS Application

##### Narrative

If he has not already done so, Barry downloads the latest changes to his laptop.

As the PLC is down, Barry is unable to check the latest version of the PLC software. As a result, Barry downloads the latest software modifications to the PLC.

In order to get the plant up and running, Barry frigs the code in certain places.

After downloading, Barry carries out a high level check on the site to validate the software download was successful. This check involves co-ordinating with Rick.

Finally, Barry checks that Rick is happy with the work carried out. Barry then retrospectively creates the job, and backs up the software.

## Modify SCADA HMI software

### Objective

Following a hardware failure, re-install the latest SCADA HMI software in order to get the system and, as a result, the plant, up and running.

### Environments

#### Day

##### Dependencies

Barry checks the job details on his workaround. Barry has discussed the changes with Rick, as the job details he has are not sufficient to carry out the change.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Barry | Hours or longer | Hourly or more | High | Medium |

##### Assets

Assets

Asset

SCADA Workstation

Laptop

STCS Application

##### Narrative

Barry copies the latest HMI software from his laptop to the SCADA workstation, and modifies the HMI software in order to make the required screen changes. Barry also changes the configuration parameters.

After the changes have been made, Barry backs up the SCADA HMI software configuration.

#### Night

##### Dependencies

Barry contacted over the phone by Rick, who verbally issues the job details. If Barry thinks a backup may not be available on site, he downloads the latest version of the SCADA software to his laptop.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Barry | Hours or longer | Hourly or more | High | High |

##### Assets

Assets

Asset

SCADA Workstation

Laptop

STCS Application

##### Narrative

Barry copies the latest HMI software from his laptop to the SCADA workstation. Barry then frigs the configuration parameters to ensure the plant remains monitored.

Once the SCADA is up and running, Barry re-generates his own job and inputs information about the work he did in order to re-install the SCADA HMI software, and backs up the SCADA HMI software configuration files.

## Resolve reservoir alarm

### Objective

Resolve operational alarm due to high reservoir water level.

### Environments

#### Day

##### Dependencies

Rick is currently in the control room, and hears an alarm.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Rick | Minutes | Daily - Weekly | Low | None |

##### Assets

Assets

Asset

SCADA Workstation

ICT PC

STCS Application

##### Narrative

Rick looks at the SCADA monitor nearest to him and notices that the levels of the reservoir nearby are unusually high. When the level gets too high, the entire works need to be shutdown.

In this situation, Rick knows exactly what to do. After stopping the alarm, Rick logs into the ICT PC next to the SCADA workstation, and clicks on the TMSview icon. After logging into TMSview, he finds the location of a pumping station 10 miles upstream on the map and connects to it. After a few moments, he masters the main pump before switching it off. Rick then returns the pump to its normal slave setting before shutting down TMSview. The alarm periodically starts and stop again but, after about an hour, the reservoir level normalises again.

#### Night

##### Dependencies

Rick is currently in the control room, and hears an alarm.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Rick | Hours or longer | Daily - Weekly | High | Low |

##### Assets

Assets

Asset

SCADA Workstation

ICT PC

STCS Application

##### Narrative

Rick looks at the SCADA monitor nearest to him and notices that the levels of the reservoir nearby are unusually high. When the level gets too high, the entire works need to be shutdown.

In this situation, Rick knows exactly what to do. After stopping the alarm, Rick logs into the ICT PC next to the SCADA workstation, and clicks on the TMSview icon. After logging into TMSview, he finds the location of a pumping station 10 miles upstream on the map and connects to it. After a few moments, he masters the main pump before switching it off. Rick then returns the pump to its normal slave setting before shutting down TMSview. The alarm periodically starts and stop again but, after about an hour, the reservoir level normalises again.

## Take chemicals delivery

### Objective

Take delivery of a scheduled order of chemicals

### Environments

#### Day

##### Dependencies

A chemicals order has been made. Rick is currently logged into the SCADA and ICT PCs in the control room. Rick is the only person available to take the delivery.

##### Personas

Personas

| Persona | Duration | Frequency | Demands | Goals |
| --- | --- | --- | --- | --- |
| Rick | Minutes | Daily - Weekly | Low | Low |

##### Assets

Assets

Asset

Works Diary

##### Narrative

At 10am, Rick receives a call from the main gate intercom. Rick looks at the CCTV camera, and sees the regular sight of the chemical deliveries truck. After a brief conversation, Rick deduces that the delivery is authentic and opens the main gate.

Rick leaves the control room and heads down to the loading area. Once there, he supervises the delivery. After 20 minutes, the Chlorine delivery has been unloaded and signed for, and Rick heads back to the control room. When he gets back, he sits down and note the time of the Chlorine delivery in the works diary.

# Use Cases

## Overview

This chapter describes, in more abstract terms than tasks, the interaction which takes place between roles and the system. Use Cases operationalise goals and requirements, and each step in a use case may lead to possible exceptions. These exceptions may be related to obstacles.

## Authenticate with ICT services

### Actors

- Plant Operator

### Description

Authenticate with ICT services

### Environments

#### Day

##### Pre-conditions

Plant Operator not authenticated with ICT services

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Plant operator enters credentials. | None |
| 2 | System verifies credentials. | None |

##### Post-conditions

Plant Operator authenticated with ICT services

#### Night

##### Pre-conditions

Plant Operator not authenticated with ICT services

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Plant operator enters credentials. | None |
| 2 | System verifies credentials. | None |

##### Post-conditions

Plant Operator authenticated with ICT services

## Log Event

### Actors

- Plant Operator

### Description

Anomalous event logged.

### Environments

#### Day

##### Pre-conditions

Alarm raised.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Plant operator enters details of notable event. | None |
| 2 | System acknowledges log entry. | None |

##### Post-conditions

Notable event logged.

#### Night

##### Pre-conditions

Alarm raised.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Plant operator enters details of notable event. | None |
| 2 | System acknowledges log entry. | None |

##### Post-conditions

Notable event logged.

## Modify Telemetry Software

### Actors

- Instrument Technician

### Description

Modify telemetry software

### Environments

#### Day

##### Pre-conditions

Software repository online. Alarm mechanisms online. Instrument technician authenticated with outstation. Instrument technician authenticated with telemetry network. Modified software on laptop and outstation.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Instrument Technician requests verification of software change | - No changes |
| 2 | System sends software change alarm to Telemetry Network | None |
| 3 | System displays modified software changes to Instrument Technician | None |
| 4 | Instrument Technician submits software modification report to the system. | - Unavailable software repository |
| 5 | System acknowledges software modification. | None |

##### Post-conditions

Modified software alarm received by telemetry network. Modified software on software repository. Software modification report on software repository.

#### Night

##### Pre-conditions

Alarm mechanisms online. Instrument technician authenticated with outstation. Modified software on laptop and outstation.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Instrument Technician requests verification of software change | None |
| 2 | System sends software change alarm to Telemetry Network | None |
| 3 | System displays modified software changes to Instrument Technician | None |

##### Post-conditions

Modified software alarm received by telemetry network.

## Raise Alarm

### Actors

- Machine

### Description

Raise alarm from equipment

### Environments

#### Day

##### Pre-conditions

Alarm mechanisms online. Machine in anomalous state.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Machine submits alarm to telemetry network. | None |
| 2 | System acknowledges alarm submission. | None |

##### Post-conditions

Software alarm received by telemetry network

#### Night

##### Pre-conditions

Alarm mechanisms online. Machine in anomalous state.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Machine submits alarm to telemetry network. | None |
| 2 | System acknowledges alarm submission. | None |

##### Post-conditions

Software alarm received by telemetry network

## Suppress Alarm

### Actors

- Plant Operator

### Description

suppress raised alarm

### Environments

#### Day

##### Pre-conditions

Audible alarm raised.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Plant operator sends suppress request. | None |
| 2 | System silences audio and visual alarms. | None |

##### Post-conditions

Audible alarm silenced.

#### Night

##### Pre-conditions

Audible alarm raised.

##### Steps

Steps

| Step | Description | Exceptions |
| --- | --- | --- |
| 1 | Plant operator sends suppress request. | None |
| 2 | System silences audio and visual alarms. | None |

##### Post-conditions

Audible alarm silenced.

# Data Flows

## Models

### Day

## Day

Data Flows

| Name | From | Type | To | Type | Assets | Obstacles |
| --- | --- | --- | --- | --- | --- | --- |
| alarm notification | Raise Alarm | process | SCADA Workstation | datastore | - PLC Event | None |
| alarm override | Suppress Alarm | process | SCADA Workstation | datastore | - Suppress Event | None |
| anomaly notes | Plant Staff | entity | Log Event | process | - Diary Entry | None |
| diary entry | Log Event | process | Works Diary | datastore | - Diary Entry | None |
| failure event | PLC | entity | Raise Alarm | process | - PLC Event | None |
| manual alarm override | Plant Staff | entity | Suppress Alarm | process | - Suppress Event | None |

# Goals

## Overview

This chapter describes the goals that the planned system needs to satisfy. These are subsequently realised by requirements and tasks, but may be obstructed by obstacles.

## Models

### Day

### Night

## Access Control

Access Control attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Secure access control. | None | None |

## Access revocation

Access revocation attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Procedures and assurance mechanisms for enabling and disabling remote access connections shall be implemented. | TBC | Standard 7.6(qq) |

## ACME Change Control

ACME Change Control attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Existing ACME Change Control procedures to co-ordinate RFCs shall be followed | None | Standard 7.1(n) |

## Active Directory Network Services

Active Directory Network Services attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Network services shall be controlled using Active Directory | Network connectivity to any SCADA, Telemetry, and Control System application workstation can be cut using Active Directory. | Standard 7.2(q) |

## Active Directory software deployment

Active Directory software deployment attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | SCADA, Telemetry and Control System software shall be deployed via Active Directory. | Automated alerts fire from the Active Directory domain controller when the software has been installed. | Standard 7.2(r). Relying on Active Directory to disallow installations over logged installation. |

## Anomalous credentials reset

Anomalous credentials reset attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Night | Maintain | Medium | Prior to responding to a password reset request, service desk operators shall monitor recent login logs for any malicious activity, and report any anomalies to the Information Security Manager. | Login attempt logs available to service desk, together with a procedure for contacting the Information Security Manager in the event of anomalies. |  |

## Anti-virus

Anti-virus attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Malware (Anti-Virus) software shall be deployed on workstations, mobile devices, and servers. | TBC | Standard 7.11(uuu) |

## Asset Inventory

Asset Inventory attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | All systems and components shall be documented to ensure an up-to-date Asset Inventory is maintained. | None | Standard 7.14(oooo) |

## Authorised removable media

Authorised removable media attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Use of removable media shall be authorised prior to use with SCADA, Telemetry, and Control System workstations. | TBC | Standard 7.10(ooo) |

## Backup hardware

Backup hardware attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Hardware used to restore backups shall be retained to ensure the availability of data. | None | Standard 7.13(iiii) |

## Backup logs

Backup logs attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Security, System, and Application event logs shall be included in the backup activity. | None | Standard 7.13(jjjj) |

## Backup media

Backup media attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | SCADA, Telemetry, and Control System software shall be copied to removable media. | None | Standard 7.13(eeee) |

## Backup storage

Backup storage attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Backups shall be stored in a fireproof safe. | None | Standard 7.13(hhhh) |

## Backup testing

Backup testing attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Backups shall be regularly tested. | Regular backup test to check integrity | Standard 7.13(ffff) |

## Backup transmission

Backup transmission attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Backups shall be transported / transmitted securely. | None | Standard 7.13(hhhh) |

## Build data

Build data attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | All build and configuration details shall be documented. | None | Standard 7.14(pppp) |

## Business activity asset

Business activity asset attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Users shall only use information assets and the ICT infrastructure for the purpose of ACME business activities. | None | Organisational InfoSec Policy 2.5 para 1. |

## Compensating controls

Compensating controls attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Where patches cannot be deployed, compensating controls shall be applied to mitigate vulnerabilities remedied by undeployable patches. | TBC | Standard 7.11(ttt) Standard 7.11(vvv) |

## Compliance audit

Compliance audit attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Regular audits shall be carried out to ensure the ACME Information Security Policy is being followed. | None | Standard 7.14 |

## Computer Rooms

Computer Rooms attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Server / Master level SCADA, Telemetry, and Control Systems shall be secured in restricted access computer rooms. | TBC | Standard 7.15(uuuu) |

## Control testing

Control testing attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Patches and compensating controls shall be tested prior to release. | When a patch is released, the written process log notes the testing carried out and the outcome. | Standard 7.11(sss) |

## Critical area logging

Critical area logging attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Access to secure critical areas and systems shall be logged. | None | Standard 7.12(aaaa) |

## Critical Vendor patches

Critical Vendor patches attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Critical Vendor Patches shall be installed within their recommended timescales. | None | Standard 7.11(ppp) |

## DetectUnknown applications

DetectUnknown applications attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Night | Detect | Medium | The system shall detect vulnerability Unknown applications after its occurrence. | None | None |

## DetectWar-dialing

DetectWar-dialing attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Night | Detect | Medium | The system shall detect threat War-dialing after its occurrence. | None | None |

## DetectWar-dialing attack

DetectWar-dialing attack attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Night | Detect | Medium | The system shall detect risk War-dialing attack after its occurrence. | None | None |

## EnterpriseSCADA access control

EnterpriseSCADA access control attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | EnterpriseSCADA access control credentials shall meet ACME security policy requirements. | TBC | Standard 7.8(ccc) |

## Environmental controls

Environmental controls attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Environmental controls shall be installed to ensure vendor operating parameters are met. | TBC | Standard 7.15(yyyy) |

## Fire detection

Fire detection attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Fire detection mechanisms shall be installed. | TBC | Standard 7.15(zzzz) |

## Fire suppression

Fire suppression attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Fire suppression systems shall be installed. | TBC | Standard 7.15(aaaaa) |

## Firewall and DMZ

Firewall and DMZ attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | All connections between SCADA, Telemetry and Control Systems shall be protected between internal boundaries and private/public domains by the application of a Firewall and DMZ | None | Standard 7.2(u) Standard 7.3(x) |

## Firewall rule attributes

Firewall rule attributes attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Firewall rules shall contain the following attributes: device address, path (source to target system), port, protocol, user, device name | None | Standard 7.3(aa) |

## Firewall trafffic monitoring

Firewall trafffic monitoring attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Firewall traffic shall be monitored by the existing Firewall administration software. | None | Standard 7.3(bb) Standard 7.3(ee) |

## ICT awareness

ICT awareness attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | All ICT partner personnel shall be made aware of the requirements for the proper handling and maintenance of the equipment hosted in the secure areas. | TBC | Organisational InfoSec Policy 2.9 para 4. |

## Id level access control

Id level access control attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | User id level access control shall be enabled by Active Directory for all administrative activities on all servers and workstations. | TBC | Standard 7.8(xx) Standard 7.8(zz) |

## Identifiable connections

Identifiable connections attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | All connections to SCADA, Telemetry, and Control Systems shall be identified | All network traffic can be tracked to registered, authorised, network point. | Standard 7.2(s) |

## Important Vendor patches

Important Vendor patches attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Important Vendor patches shall be installed where applicable. | TBC | Standard 7.11(rrr) |

## Industrialised secure cabinets

Industrialised secure cabinets attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | PLC, SCADA, and Control Systems in operational environments shall be situated in industrialised secure cabinets. | Local operators periodically check security of cabinets. | Standard 7.15(wwww) |

## Internet access

Internet access attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Internet access from SCADA, Telemetry and Control Systems shall be disabled. | None | Standard 7.7(uu) |

## Intrusion Prevention System

Intrusion Prevention System attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | An Intrusion Prevention System shall be deployed to monitor and respond to malicious traffic. | None | Standard 7.2(v) |

## Key ICT security

Key ICT security attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | The area used to host key onsite ICT equipment shall be physically secured to prevent unauthorised physical access to any equipment used to process, store, or transmit ACME information. | TBC | Organisational InfoSec Policy 2.9 para 2. |

## Least privilege connections

Least privilege connections attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Firewalls shall block all network traffic unless it has been explicitly allowed. | TBC | Standard 7.2(v) |

## Locked cabinets

Locked cabinets attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Server / Master level SCADA, Telemetry, and Control Systems shall be installed in locked cabinets. | TBC | Standard 7.15(vvvv) |

## Locked doors

Locked doors attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Physical security measures at sites shall include the use of locked doors. | TBC | Standard 7.15(xxxx) |

## Log file overwrite

Log file overwrite attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Log files shall be set to overwrite. | None | Standard 7.12(cccc) |

## Log file size

Log file size attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Log file sizes shall be set to a minimum of 500 MB. | None | Standard 7.12(bbbb) |

## Malware data scanning

Malware data scanning attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Low | All data shall be scanned for Malware | None | Standard 7.11(xxx) |

## Offsite backup

Offsite backup attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Backups shall be stored offsite to ensure their availability in the event of a localised disaster. | None | Standard 7.14(gggg) |

## Operating system logs

Operating system logs attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Low | Operating system activity shall be logged. | Logging for future analysis. | Standard 7.12(zzz) |

## OS hardening

OS hardening attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | SCADA, Telemetry, and Control System operating systems shall run only those services necessary for running of installed SCADA, Telemetry, and Control System applications. | TBC | Standard 7.10(kkk) Standard 7.10(mmm) |

## Password expiration

Password expiration attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Passwords shall be set to expire in line with ACME policy. | TBC | Standard 7.9(fff) |

## Password length

Password length attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Passwords shall be set to a minimum of 12 characters. | TBC | Standard 7.9(eee) |

## Password mixed characters

Password mixed characters attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Passwords shall contain a mixture of upper, lower, and alphanumeric characters. | None | Standard 7.9(ggg) |

## Password policy

Password policy attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Pasword policy | TBC | None |

## Password storage

Password storage attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Passwords shall not be written down. | None | Standard 7.9 (jjj) |

## Patch Risk Management

Patch Risk Management attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Where patching adversely impacts the availability, integrity, or stability of operational systems, a risk assessment shall be conducted and recommendations approved by the information security manager, the Standards Committee and system owner. | Risk and impact assessment fields fields filled in for each approved patch. | Standard 7.11(qqq) |

## Power provision

Power provision attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Appropriate power provision shall be in situ to meet operational requirements. | TBC | Standard 7.15(bbbbb) |

## Product security accreditation

Product security accreditation attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Achieve | High | SCADA, Telemetry, and Control System products shall be accredited prior to their installation to the Enterprise SCADA environment. | None | Standard 7.1(p) |

## Remote access inventory

Remote access inventory attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | An inventory of all authorised remote access connections shall be maintained. | TBC | Standard 7.6(nn) |

## Remote connection audit

Remote connection audit attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Regular audits shall be carried out ensuring that no unauthorised remote access connections exist. | None | Standard 7.14(nnnn) |

## Router Installation

Router Installation attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Routers shall be installed in compliance with the TCP/IP addressing standard, the ACME firewall build, and the ACME VPN configuration standard. | TBC | Standard 7.4(ff) |

## Router maintenance

Router maintenance attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Routers shall be maintained in compliance with the TCP/IP addressing standard, the ACME firewall build, and the ACME VPN configuration standard. | TBC | Standard 7.4(ff) |

## Scanned media

Scanned media attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | All removeable media shall be scanned prior to insertion into target systems. | None | Standard 7.11(www) |

## Secure backup

Secure backup attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | SCADA, Telemetry, and Control System software and configuration data shall be backed-up to ensure swift recovery. | None | Standard 7.13(dddd) |

## Secure changes

Secure changes attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Changes or additions to SCADA, Telemetry and Control Systems shall be secure. | None |  |

## Secure network infrastructure

Secure network infrastructure attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Safeguard network infrastructure | None | None |

## Secure operating environment

Secure operating environment attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | The operating environment for SCADA, Telemetry and Control Systems associated with assets owned and operated by ACME shall be secure. | None | None |

## Secure Operating Systems

Secure Operating Systems attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Secure OS | None |  |

## Secure Remote Access

Secure Remote Access attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Secure Remote access | None | Standard 7.6 |

## Secure Roles

Secure Roles attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | The security responsibilities of ACME employees, partners, and contractors responsible for installing, configuring, implementing, and operating ICT infrastructure supporting SCADA, Telemetry, and Control Systems shall be specified and be specified and communicated to all relevant parties. | None | Standard 5(a) |

## Secure Routers

Secure Routers attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Routers shall be installed and maintained in compliance with the TCP/IP addressing standard, the ACME firewall build, and the ACME VPN configuration standard. | None | None |

## Secure site

Secure site attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | SCADA, Telemetry, and Control Systems shall be physically protected from malicious attack, unauthorised access, and theft. | None | Standard 7.15(tttt) |

## Secure SLA

Secure SLA attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | ICT Service Level Agreements for sites with SCADA, Telemetry, and Control Systems shall be commensurate with the criticality of the site. | TBC | Standard 7.1(o) |

## Secure wifi

Secure wifi attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Wireless systems shall be securely deployed. | None | Standard 7.5 |

## Standard build

Standard build attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | All SCADA, Telemetry, and Control Systems operating systems shall be installed from a standard ACME build image. | None | Standard 7.1 (l) |

## Third party compliance

Third party compliance attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Security responsibility shall be written into 3rd party agreements. | TBC | Standard 7.14(rrrr) |

## TMS access control

TMS access control attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | TMS access control credentials shall meet ACME security policy requirements. | TBC | Standard 7.8(ddd) |

## User Id Least privilege

User Id Least privilege attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | User id access shall be based on least privilege. | None | Standard 7.8(aaa) |

## Vendor passwords

Vendor passwords attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | Medium | Default vendor passwords shall be changed prior to deployment of system. | TBC | Standard 7.9(hhh) |

## Versioned PLC changes

Versioned PLC changes attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | PLC software changes shall be versioned. | This can be achieved by using a centralised software repository. | Standard 7.14(ssss) |

## Warranted Access

Warranted Access attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Credentials to SCADA, Telemetry, and Control Systems shall be authorised based on current operational need. | TBC | Standard 7.8(ww) |

## Warranted connection

Warranted connection attributes

| Environment | Category | Priority | Definition | Fit | Issues |
| --- | --- | --- | --- | --- | --- |
| Day | Maintain | High | Each SCADA, Telemetry, and Control System network connection shall be warranted. | The warrant for each SCADA, Telemetry, and Control System connection is recorded in the BMS. | Standard 7.2(t) |

# Responsibilities

## Overview

This chapter describes the dependencies between roles, and the various artifacts that roles are responsible for.

## Models

### Day

### Night

## Dependencies

### Day

Dependency

| Depender | Dependee | Type | Dependency | Rationale |
| --- | --- | --- | --- | --- |
| Information Security Manager | Vendor | goal | Product security accreditation | ACME rely on vendors to provide proof of a product's level of security. |
| Information Security Manager | ICT Partner | goal | Active Directory software deployment | Standard 6(f) |
| Information Security Manager | ICT Partner | goal | Router Installation | Standard 6(h) |
| Information Security Manager | ICT Partner | goal | Router maintenance | Standard 6(h) |
| Information Security Manager | Plant Operator | goal | Industrialised secure cabinets | Limited security team |

### Night

Dependency

| Depender | Dependee | Type | Dependency | Rationale |
| --- | --- | --- | --- | --- |
| Information Security Manager | Service Desk | goal | Anomalous credentials reset | Goal relies on service desk staff being switch-on enough to check logs, and pro-active enough to contact the ACME information security team. |

# Obstacles

## Overview

Obstacles are obstructions to goals. These may be refined to possible threats and vulnerabilities.

## Models

### Day

## Accessible equipment

Accessible equipment attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Accessible equipment |

## Crafted packets

Crafted packets attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Integrity Threat | Crafted packets |

## Desktop installation

Desktop installation attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Confidentiality Threat | Desktop installation |

## DLL preloading

DLL preloading attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Integrity Threat | DLL preloading |

## Exposed ICT cabinets

Exposed ICT cabinets attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Exposed ICT cabinets |

## Incomplete firewall rule-set

Incomplete firewall rule-set attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Incomplete firewall rule-set |

## Malicious remote installation

Malicious remote installation attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Accountability Threat | Malicious remote installation |

## Malware application

Malware application attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Integrity Threat | Malware application |

## Redundant application

Redundant application attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Confidentiality Threat | Redundant application |

## Scanning software disabled

Scanning software disabled attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Scanning software disabled |

## Scanning software not installed

Scanning software not installed attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Scanning software not installed |

## Software incompatibility

Software incompatibility attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Software incompatibility |

## Surplus equipment

Surplus equipment attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Surplus equipment |

## Ubiquitous identity data

Ubiquitous identity data attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Ubiquitous identity data |

## Unavailable software repository

Unavailable software repository attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Availability Threat | Active Directory is unavailable. |

## Undetected malicious traffic

Undetected malicious traffic attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Undetected malicious traffic |

## Unsanitised equipment

Unsanitised equipment attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Unsanitised equipment |

## Unscanned media

Unscanned media attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Unscanned media |

## Unused PC application

Unused PC application attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Unused PC application |

## Vendor passwords unchanged

Vendor passwords unchanged attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Integrity Threat | Vendor passwords unchanged |

## Zero-day threat exploit

Zero-day threat exploit attributes

| Environment | Category | Definition |
| --- | --- | --- |
| Day | Vulnerability | Zero-day threat exploit |

# Vulnerabilities

## Overview

This chapter describes the vulnerabilities evident in this system. These vulnerabilities may arise due to the nature of the environment, the system itself, or some aspect of the system's design.

## Vulnerability Types

Vulnerability Types

| Type | Description |
| --- | --- |
| Configuration | A vulnerability resulting from an error in the configuration and administration of a system or component. |
| Design | A vulnerability inherent in the design or specification of hardware or software whereby even a perfect implementation will result in a vulnerability. |
| Implementation | A vulnerability resulting from an error made in implementing software or hardware of a satisfactory design. |

## Vulnerability Severities

Vulnerability Severities

| Type | Description |
| --- | --- |
| Negligible | Lack of services for reporting purposes. No impact on quality or performance. |
| Marginal | Lack of service for a certain time. |
| Critical | Lack of services for a certain time frame, impacting water supply to enhanced sites, e.g. hospitals. |
| Catastrophic | Leads to death of one or more people, e.g. latent introduction of Cryptosporidium into the water supply. |

## Exposed cabinets

### Type

Implementation

### Description

Equipment cabinets are easily accessible to non-authorised personnel

### Environments

#### Day

##### Severity

Critical

##### Assets

Assets

Asset

ICT cabinet

#### Night

##### Severity

Negligible

##### Assets

Assets

Asset

ICT cabinet

## Incompatible security controls

### Type

Configuration

### Description

Security software is incompatible with SCADA, Telemetry, or Control System application

### Environments

#### Day

##### Severity

Marginal

##### Assets

Assets

Asset

SCADA Workstation

Laptop

ICT PC

#### Night

##### Severity

Marginal

##### Assets

Assets

Asset

SCADA Workstation

Laptop

ICT PC

## Incomplete firewall rules

### Type

Configuration

### Description

Firewall rule set fails to identify an intentional attack.

### Environments

#### Day

##### Severity

Marginal

##### Assets

Assets

Asset

Firewall

#### Night

##### Severity

Marginal

##### Assets

Assets

Asset

Firewall

## Incomplete Intrusion Detection rules

### Type

Configuration

### Description

Intrusion detections rules do not cover possible malicious usage of network applications.

### Environments

#### Day

##### Severity

Critical

##### Assets

Assets

Asset

Intrusion Detection System

#### Night

##### Severity

Critical

##### Assets

Assets

Asset

Intrusion Detection System

## Redundant hardware

### Type

Implementation

### Description

Unused PC hardware left lying around may contain sensitive data or credentials

### Environments

#### Day

##### Severity

Marginal

##### Assets

Assets

Asset

ICT PC

#### Night

##### Severity

Marginal

##### Assets

Assets

Asset

ICT PC

## Ubiquitous identity and knowledge

### Type

Design

### Description

The user id component of ICT access credentials are multi-use, and this knowledge is commonly known and, occasionally, shared.

### Environments

#### Day

##### Severity

Marginal

##### Assets

Assets

Asset

ICT credentials

#### Night

##### Severity

Critical

##### Assets

Assets

Asset

ICT credentials

## Unchanged vendor passwords

### Type

Configuration

### Description

Passwords used to access operating systems resources are unchanged.

### Environments

#### Day

##### Severity

Critical

##### Assets

Assets

Asset

STCS Application

#### Night

##### Severity

Critical

##### Assets

Assets

Asset

STCS Application

## Unknown applications

### Type

Implementation

### Description

PC and PC applications with no clear provenance may be more liable to exploitation, and used as an attack vector for obtaining sensitive data or credentials

### Environments

#### Day

##### Severity

Marginal

##### Assets

Assets

Asset

ICT PC

#### Night

##### Severity

Marginal

##### Assets

Assets

Asset

ICT PC

## Wide Scope

### Type

Design

### Description

Red team testers exercise discretion over such a wide scope that activities may be carried out which are legally ambiguous.

### Environments

#### Day

##### Severity

Critical

##### Assets

Assets

Asset

ICT PC

Corporate Network

Enterprise SCADA Network

ICT Application

STCS Application

# Attackers

## Overview

This chapter describes the attackers the planned system needs to defend against. These attackers will launch attacks, in the shape of threats, and take advantage of the vulnerabilities described by this specification.

## Motivations

Attacker Motivations

| Motivation | Description |
| --- | --- |
| Hactivism | The nonviolent use of illegal or legally ambiguous digital tools in the pursuit of political ends. |
| Cyber-extortion | To be defined |
| Defamation | To be defined |
| Cyber-tagging | To be defined |
| Headlines/press | To be defined |
| Data theft | To be defined |
| Data destruction | To be defined |
| Data modification | To be defined |
| System resource theft | To be defined |
| Network resource theft | To be defined |
| Revenge | To be defined |
| Improved organisational position | To be defined |
| Improved esteem | To be defined |
| Thrill-seeking | To be defined |
| Fraud | To be defined |
| Disruption | To be defined |
| Accident | To be defined |
| Indifference | To be defined |
| Money | financial gain |
| Productivity | financial gain |

## Capability

Attacker Capabilities

| Capability | Description |
| --- | --- |
| Resources/Equipment | To be defined |
| Resources/Facilities | To be defined |
| Resources/Personnel and Time | To be defined |
| Resources/Funding | To be defined |
| Technology | To be defined |
| Software | To be defined |
| Knowledge/Education and Training | To be defined |
| Knowledge/Books and Manuals | To be defined |
| Knowledge/Methods | To be defined |

## Ben

Ben is a security engineer for RedTeam LLC. He is working with a junior tester (Alex) to conduct a red team test on ACME Water; they are evaluating ACME's security posture to ensure their security policies on te use of IT are being followed.

Ben has a first degree in Computer Science, and Masters degree in Information Security. Ben is also holds CREST accreditations.

Ben has practical experience with several open source penetration testing frameworks, such as Metasploit. Based on his experience, and the experiences of his colleagues, Ben also uses a number of effective in-house network exploitation tools.

As an industrial control systems penetration tester, Ben carries out limited engagements involving the testing security of PCs and network infrastructure at critical infrastructure sites.

Ben environmental attributes

| Environment | Roles | Motives | Capabilities |
| --- | --- | --- | --- |
| Day | Cracker | Data theft Data destruction Data modification | Resources/Funding : Low Resources/Equipment : Medium Software : Medium Technology : High Knowledge/Education and Training : High Knowledge/Books and Manuals : High |
| Night | Cracker | Data theft Data destruction Data modification | Resources/Funding : Low Resources/Equipment : Medium Software : Medium Technology : High Knowledge/Education and Training : High Knowledge/Books and Manuals : High |

## Gareth

Gareth is 35, lives in Stepford and is unemployed.

Gareth has friends who work as building labourers as part of some capital project at Rick's water treatment works. They tell him that computer hardware and surplus piping and copper can be found around the site.

Gareth environmental attributes

| Environment | Roles | Motives | Capabilities |
| --- | --- | --- | --- |
| Day | Petty Criminal | System resource theft | Resources/Personnel and Time : High |
| Night | Petty Criminal | System resource theft | Resources/Personnel and Time : High |

## Unintentional Rick

Rick has a basic operating knowledge of Windows; enough to carry out his day-to-day routines, check his email, and navigate the main sections of InfoPortal.

Rick isn't a power-user; he acknowledges that younger "whizz kids" are much more adept at using PCs than he is. Consequently, he only uses a sub-set of the desktop icons currently on his desktop. "I don't know what these other applications do", Rick says,"I've tried clicking on one and it just gives me a Data Protection Warning. I guess this means I'm not authorised to use it."

Unintentional Rick environmental attributes

| Environment | Roles | Motives | Capabilities |
| --- | --- | --- | --- |
| Day | Plant Operator | Productivity | Resources/Facilities : Low Software : Low |
| Night | Plant Operator | Productivity | Resources/Facilities : Low Software : Low |

## Victor

Victor is a contractor and expert in the SCADA systems used by ACME, having helped develop them over 15 years ago.

Due to the recent economic downtime, Victor has been forced to take a recent pay-cut.

Victor environmental attributes

| Environment | Roles | Motives | Capabilities |
| --- | --- | --- | --- |
| Day | Vendor | Revenge | Technology : Medium Software : Low |

# Threats

## Overview

This chapter describes the threats which impact the planned system.

## Threat Types

Threat Types

| Type | Description |
| --- | --- |
| Electronic/DoS and DDoS | A Denial-of-Service (DoS) attack involves a malicious attempt to disrupt the operation of a computer system or network that is connected to the Internet. A Distributed Denial-of-Service (DDoS) attack is a more dangerous evolution of a DoS attack because it utilises a network of compromised zombie computers to mount the attack, so there is no identifiable single source. |
| Electronic/Hacking | Hackers want to get into your computer system and use them for their own purposes. There are many hacking tools available on the internet as well as online communities actively discussing hacking techniques enabling even unskilled hackers to break into unprotected systems. Hackers have a range of motives; from showing off their technical prowess, to theft of money, credentials or information, to cause damage. |
| Electronic/Keystroke logging | Keystroke loggers work by recording the sequence of key-strokes that a user types in. The more sophisticated versions use filtering mechanisms to only record highly prized information such as email addresses, passwords and credit card number sequences. |
| Electronic/Malware | Malware is any program or file that is harmful to a computer, the term covers viruses, worms, Trojan horses, and spyware. Malware is becoming increasingly sophisticated and can be used to compromise computers to install DOS zombie programs or other malicious programs. |
| Electronic/Phishing and Spoofing | Phishing describes a social engineering process designed to trick an organisation's customers into imparting confidential information such as passwords, personal data or banking and financial details. Most commonly these are criminal attacks but the same techniques could be used by others to get sensitive information. |
| Insider/Manipulation | Sometimes deliberate attempts are made to acquire information or access by manipulating staff by using a range of influencing techniques. This is sometimes described as social engineering, creating situations in which someone will willingly provide access to information, sites or systems to someone unauthorised to receive it. Customer facing personnel who have been trained to be helpful and informative can be particularly vulnerable to such attacks. |
| Insider/Sabotage | Saborage is often committed by a former employee seeking revenge on their employer because of a personal grudge caused by a negative work related event such as dismissal. Although it is sometimes planned well in advance, it can also be the result of an opportunistic moment. |
| Natural | Environment / Acts of Nature |
| Physical | Physical Security |

## Threat Likelihoods

Threat Likelihoods

| Type | Description |
| --- | --- |
| Incredible | Likelihood of occurrence is negligible. |
| Improbable | Very low chance. |
| Remote | Low chance. |
| Occasional | Medium chance. |
| Probable | Fairly likely. |
| Frequent | More than likely. |

## Threat Properties

Threat Properties

| Property | Description |
| --- | --- |
| None | No impact |
| Low | Water and sewage services: Excursions from Standard of Service |
| Medium | Water and sewage services: Moderate number of customers affected. Moderate degradation of service delivery. |
| High | Water and sewage services: Large number of customers affected. Widespread degration of service delivery, or failures. |

## Enumeration

Enumeration attributes

| Attribute | Description |
| --- | --- |
| Type | Electronic/Hacking |
| Method | Various activities to determine information about systems and applications on the network |

Enumeration environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Day | Probable | Ben | Integrity : Medium () Availability : High () |

## False sensor readings

False sensor readings attributes

| Attribute | Description |
| --- | --- |
| Type | Electronic/Phishing and Spoofing |
| Method | False readings are sent to SCADA workstations |

False sensor readings environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Day | Improbable | Victor | Integrity : Medium () Availability : High () |

## Footprinting

Footprinting attributes

| Attribute | Description |
| --- | --- |
| Type | Electronic/Hacking |
| Method | Various activities to determine information about the network and systems on it. |

Footprinting environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Day | Probable | Ben | Integrity : Medium () Availability : High () |

## Kit theft

Kit theft attributes

| Attribute | Description |
| --- | --- |
| Type | Physical |
| Method | Unauthorised access to site to steal ICT kit supporting plant operations |

Kit theft environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Day | Improbable | Gareth | Integrity : Medium (Gareth wants to tamper with the stolen kit, so it can be refurbished.) Availability : High (Gareth is interested in obtaining the ICT kit above all else.) |
| Night | Remote | Gareth | Integrity : Medium (Gareth wants to tamper with the stolen kit, so it can be refurbished.) Availability : High (Gareth is interested in obtaining the ICT kit above all else.) |

## Logic bomb

Logic bomb attributes

| Attribute | Description |
| --- | --- |
| Type | Insider/Sabotage |
| Method | A logic bomb incorporated into PLC software, which causes one or more device failures upon set conditions. |

Logic bomb environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Day | Improbable | Victor | Integrity : Medium () Availability : High () |

## Password enumeration

Password enumeration attributes

| Attribute | Description |
| --- | --- |
| Type | Electronic/DoS and DDoS |
| Method | Unsuccessful enumeration of password combinations |

Password enumeration environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Day | Improbable | Ben | Integrity : Medium () Availability : High () |

## Sniffing Kerberos Authorisation

Sniffing Kerberos Authorisation attributes

| Attribute | Description |
| --- | --- |
| Type | Electronic/Hacking |
| Method | Ticket Granting Server identified and KerbSniff used to pull in domain authentication information, and KerbCrack brute forces the output file |

Sniffing Kerberos Authorisation environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Day | Remote | Ben | Integrity : Medium () Availability : High () |

## Unauthorised USB usage

Unauthorised USB usage attributes

| Attribute | Description |
| --- | --- |
| Type | Electronic/Malware |
| Method | Unscanned USB stick plugged into ICT PC. |

Unauthorised USB usage environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Night | Improbable | Unintentional Rick | Integrity : Medium () Availability : High () |

## War-dialing

War-dialing attributes

| Attribute | Description |
| --- | --- |
| Type | Electronic/Hacking |
| Method | Using a modem to scan telephone numbers to other modems to dial into. |

War-dialing environmental attributes

| Environment | Likelihood | Attackers | Security Properties |
| --- | --- | --- | --- |
| Night | Occasional | Ben | Integrity : Medium () Availability : High () |

# Risks

## Overview

This chapter describes the identified risks which impact the planned system. These arise when attackers launch attacks, manifested as threats, which expose a vulnerability. A risk is only evident if both the threat and vulnerability exist in at least one environment.

## Models

### Day

### Night

## Risk categories

Risk Categories

| Category | Description |
| --- | --- |
| Negligible | No notifiable or reportable security incident. |
| Tolerable | Localised security incident, readily contained. |
| Undesirable | Localised security incident causing disruption to core departmental services. |
| Intolerable | Significant security insident causing considerable or severe disruption to services. |

## Automated account lockout

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | Password enumeration |
| Vulnerability | Wide Scope |

### Environments

#### Day

##### Severity

Tolerable

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| None | 9 |

## Manual account lockout

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | Password enumeration |
| Vulnerability | Ubiquitous identity and knowledge |

### Environments

#### Day

##### Severity

Negligible

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| None | 9 |

## Network footprinting

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | Footprinting |
| Vulnerability | Incomplete firewall rules |

### Environments

#### Day

##### Severity

Undesirable

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| None | 9 |

## Site break-in

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | Kit theft |
| Vulnerability | Ubiquitous identity and knowledge |

### Environments

#### Day

##### Severity

Negligible

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| None | 9 |

#### Night

##### Severity

Tolerable

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| None | 9 |

## Spoof telemetry

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | False sensor readings |
| Vulnerability | Incomplete Intrusion Detection rules |

### Environments

#### Day

##### Severity

Tolerable

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| Accept Spoof telemetry | 9 |

## System enumeration

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | Enumeration |
| Vulnerability | Incomplete Intrusion Detection rules |

### Environments

#### Day

##### Severity

Intolerable

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| None | 9 |

## Virus infected workstation

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | Unauthorised USB usage |
| Vulnerability | Incompatible security controls |

### Environments

#### Night

##### Severity

Negligible

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| None | 9 |

## War-dialing attack

### Components

Risks

| Property | Value |
| --- | --- |
| Threat | War-dialing |
| Vulnerability | Unknown applications |

### Environments

#### Night

##### Severity

Tolerable

##### Mitigation Scores

Responses

| Response | Score |
| --- | --- |
| Detect War-dialing attack | 9 |

# Locations

## Models

### PooleWWTW

#### Day

#### Night

## PooleWWTW

Locations

| Location | Objects | People |
| --- | --- | --- |
| PC1 | None | None |
| WCH_1 | - PC-9 : ICT PC | None |
| WCH_2 | - PC-67 : ICT PC | None |
| WCH_3 | - PC-76 : ICT PC | None |
| WCH_4 | - SWS-1 : SCADA Workstation - PC-23 : ICT PC - PC-24 : ICT PC - PC-25 : ICT PC | - Rick : Rick |
| WCH_5 | None | None |
| WCH_6 | - M12 : Modem - R18 : Router - PC-11 : ICT PC | - Barry : Barry |
| PC2 | None | None |
| MR_1 | None | None |
| MR_2 | None | None |
| MR_3 | None | None |
| MR_4 | None | None |

# Misuse Cases

## Overview

The Misuse Cases below describe how an attacker exploits each of the identified risks.

## Exploit Automated account lockout

### Components

Components

| Component | Value |
| --- | --- |
| Risk | Automated account lockout |
| Threat | Password enumeration |
| Vulnerability | Wide Scope |

### Environments

#### Day

##### Objective

Exploit vulnerabilities in ICT PC,Corporate Network,Enterprise SCADA Network,ICT Application,STCS Application to threaten SCADA Workstation,ICT PC.

##### Narrative

Ben is supervising Alex in the running of RedTeam's ICS network auditing to identify potential vulnerabilities on ACME's corporate IT network. The tool has crawled the corporate network to identify information that identifies members of staff. Unfortunately, neither Ben nor Alex realised that an account lockout policy exists and, consequently, manages to lock several arbitrary accounts out.

In the meantime, back at the plant, Rick is unable to login to the ICT PC to use the telemetry service software because his account is locked out.

## Exploit Manual account lockout

### Components

Components

| Component | Value |
| --- | --- |
| Risk | Manual account lockout |
| Threat | Password enumeration |
| Vulnerability | Ubiquitous identity and knowledge |

### Environments

#### Day

##### Objective

Exploit vulnerabilities in ICT credentials to threaten SCADA Workstation,ICT PC.

##### Narrative

Ben has obtained a list of possible pay numbers, and is enumerating possible passwords. Unfortunately, he didn't realise that an account lockout policy exists and, consequently, manages to lock several arbitrary accounts out.

In the meantime, back at the plant, Rick is unable to login to the ICT PC to use the telemetry service software because his account is locked out.

## Exploit Network footprinting

### Components

Components

| Component | Value |
| --- | --- |
| Risk | Network footprinting |
| Threat | Footprinting |
| Vulnerability | Incomplete firewall rules |

### Environments

#### Day

##### Objective

Exploit vulnerabilities in Firewall to threaten Firewall,Corporate Network,Enterprise SCADA Network.

##### Narrative

One of Ben's colleagues is spidering web pages to find information about ACME partners, access points to portals, and various other tid-bits of open source intelligence. Periodically, he compiles a list of interesting sites and addresses, which he passes onto Ben.

Ben carries out Whois and DNS enumeration to find interesting IP addresses and, based on this, interrogates ACME DNS servers to find interesting systems. He also runs a script which trace-routes IP addresses and outputs the data to a file, which he visualises using GraphViz.

After a while, Ben has developed an access path diagram diagram of the ACME network.

## Exploit Site break-in

### Components

Components

| Component | Value |
| --- | --- |
| Risk | Site break-in |
| Threat | Kit theft |
| Vulnerability | Ubiquitous identity and knowledge |

### Environments

#### Day

##### Objective

Exploit vulnerabilities in ICT credentials to threaten SCADA Workstation,ICT PC.

##### Narrative

Gareth and two of his friends borrow some hard-hats, fluorescent vests, and ,armed with a borrowed pay-number, drive to the site in a borrowed white-van.

As promised, the certain digits of the pay number double as the gate access code. On entering the site, Gareth parks up near a remote building, which contains a small pump and a panel PC. Within a few minutes, the door lock has been broken, and the pump and PC are in the back of the van.

Meanwhile, back in the control room, Rick notices an alarm from the pump in building which has just been burgled. Rick glances up at the CCTV and sees nothing, so he walks down to the building to take a look. On his way, Gareth and his friends drive away from the building and out of the site.

#### Night

##### Objective

Exploit vulnerabilities in ICT credentials to threaten SCADA Workstation,ICT PC.

##### Narrative

Gareth and six of his friends borrow some hard-hats, fluorescent vests, and, armed with a borrowed pay-number, drive to the site in a borrowed white-van. Half a mile away from the site, Gareth parks the van in a lay-by and, together, the group walks cross-country to the site.

As promised, the certain digits of the pay number double as the gate access code. After Gareth quickly enters the code, they all enter the site, and make their way towards a remote building; this contains a small pump and a panel PC. Within a few minutes, the door lock has been broken, and the pump and PC are being carried back to the main gate.

Meanwhile, back in the control room, Rick notices an alarm from the pump in building which had just been burgled. Rick glances up at the CCTV and sees nothing, so he walks down to the building to take a look. On his way, Gareth and his friends make their way out of the site with the stolen equipment, and walk briskly across a field towards their van.

## Exploit Spoof telemetry

### Components

Components

| Component | Value |
| --- | --- |
| Risk | Spoof telemetry |
| Threat | False sensor readings |
| Vulnerability | Incomplete Intrusion Detection rules |

### Environments

#### Day

##### Objective

Exploit vulnerabilities in Intrusion Detection System to threaten SCADA Workstation,EnterpriseSCADA Server.

##### Narrative

Using his credentials, Victor connects to an EnterpriseSCADA server at a remote site. Victor customises the alarms so that certain critical warnings are ignored. Victor then proceeds to make several malicious modifications to the PLCs at that site.

## Exploit System enumeration

### Components

Components

| Component | Value |
| --- | --- |
| Risk | System enumeration |
| Threat | Enumeration |
| Vulnerability | Incomplete Intrusion Detection rules |

### Environments

#### Day

##### Objective

Exploit vulnerabilities in Intrusion Detection System to threaten SCADA Workstation,EnterpriseSCADA Server,ICT Application,STCS Application.

##### Narrative

Using the information obtained from his network footprinting exercise, Ben proceeds to obtain more information about the systems on the ACME network.

This includes banner grabbing to obtain data about OS and application versions, and enumerating HTTP to find possible vulnerabilities.

Because Ben knows legacy SCADA applications are being used, he also enumerates posssible RPC and NetBios name servers and sessions.

Having heard stories about the Stuxnet virus, he decides to enumerate possible SQL Server instances as well.

## Exploit Virus infected workstation

### Components

Components

| Component | Value |
| --- | --- |
| Risk | Virus infected workstation |
| Threat | Unauthorised USB usage |
| Vulnerability | Incompatible security controls |

### Environments

#### Night

##### Objective

Exploit vulnerabilities in SCADA Workstation,Laptop,ICT PC to threaten ICT PC,ICT Application,STCS Application.

##### Narrative

Rick's son puts a pirated copy of Avatar on a USB stick for his father to watch at work. Rick keeps the USB stick in his coat pocket and forgets about it until, three weeks later, Rick is unworking an unusually quiet shift and remembers the USB stick.

Rick sits down in front of the backup ICT PC, plugs in the USB stick, and copies over the file. Unknown to Rick, the USB stick also contains malware, which executed after being plugged into the PC, which caused the introduction of a number of malicious DLLs shared with the telemetry service software, changed the account's environment variables and PATH settings.

## Exploit War-dialing attack

### Components

Components

| Component | Value |
| --- | --- |
| Risk | War-dialing attack |
| Threat | War-dialing |
| Vulnerability | Unknown applications |

### Environments

#### Night

##### Objective

Exploit vulnerabilities in ICT PC to threaten SCADA Workstation,ICT PC,Modem.

##### Narrative

Ben has obtained main extension numbers for water-treatment works, together with possible ranges. As soon as 6pm has passed, Ben uses his war-dialing tools (PhoneSweep, a number a custom DOS scripts, and some information about default credentials that is maintained at www.phenoelit-us.org/dpl/dpl.html) to identify and penetrate candidate modem.

After about an hour, Ben finds the number for a modem and, using his tool manages to both deduce information about both the modem and the connected operating system. He also manages to connect into the PC, which appears to be an engineering workstation used for updating PLC software and has been left logged in.

Ben promptly installs a keylogger onto this PC before logging out.

# Responses

## Accept

### Accept Spoof telemetry

#### Risk

Spoof telemetry

#### Cost

Accept Spoof telemetry Costs

| Environment | Cost |
| --- | --- |
| Day | Low |
| Night | Low |

#### Rationale

Accept Spoof telemetry Rationale

| Environment | Rationale |
| --- | --- |
| Day |  |
| Night |  |

## Mitigate

### Detect War-dialing attack

#### Risk

War-dialing attack

#### Type

Detect War-dialing attack Type

| Environment | Roles |
| --- | --- |
| Night | Detect : detected After |

# Functional Requirements

## Functional Requirements

### Corporate Network

Corporate Network Functional Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| CN-1 | Access control to wifi devices shall be managed through Active Directory. | 1 | Active Directory enforces access control for ACME corporate devices. | A wifi device is added to the corporate network but not to Active Directory. Access to a resource in Active Directory fails. | Standard 7.5(mm) |
| CN-3 | Email from SCADA, Telemetry and Control Systems shall be routed through the corporate e-mail System. | 1 | Email services are managed by corporate IT. |  | Standard 7.7 (vv) |

### Firewall

Firewall Functional Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| F-5 | Firewalls shall be implemented using dedicated hardware. | 1 | Separate firewall appliances remove the risk of less critical hardware affecting network availability if this hardware hosts the firewall. | Firewall software fails to run if any STCS or corporate applications are running on the same device. | Standard 7.3(y) |

### ICT Application

ICT Application Functional Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| ICTA-1 | All users shall have read and signed the Acceptable Use Policy (AUP) and Code of Conduct (CoCo) forms. | 1 | None | None | Standard 7.14(mmmm) |

### ICT credentials

ICT credentials Functional Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| IC-1 | Administration level privileges shall be approved using the Code of Conduct (CoCo) forms on InfoPortal. | 1 | None | Group access for administrators is monitored manually, and each instance of access is logged. | Standard 7.8(bbb) |

### InfoPortal

InfoPortal Functional Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| IZ-1 | When staff responsibilities and applicability is unclear, the Information Security manager shall be consulted for advice. | 1 | None | None | Standard 5(b) |

### Router

Router Functional Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| R-1 | All ACME employees responsible for maintaining information security measures within their area of responsibility shall ensure routing devices are not compromised through misuse of other information processing facilities. | 1 | None | Generic users and passwords are not present on any router devices. For any given router under a particular contract, the contract states how service availability is maintained and what the partner's security policy expectations are, including explicit definitions of security requirements. | Standard 7.4(hh) |

### STCS Application

STCS Application Functional Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| STCS-1 | All third party requests for remote access to SCADA, Telemetry, and Control Systems may be reviewed using appropriate processes. | 1 | None | None | Standard 7.6(rr) |
| STCS-2 | PLC and SCADA application programs shall be stored in the central software repository. | 1 | All software stored in repository. | Create new PC image. Download app | Standard 7.14(kkkk) |

# Nonfunctional Requirements

## Security Requirements

Requirements

| Id | Description | Priority | Rationale | Fit Criterion | Originator |
| --- | --- | --- | --- | --- | --- |
| CN-2 | Where vendor installed communication devices exist, backdoor access shall not be configured. | 1 | Backdoors are vulnerabilities | Network scans find no backdoor access. | Standard 7.6(tt) |
| CN-4 | Connecting PLCs and RTUs shall be port-scanned to determine their requisite ports, services, and protocols. | 1 | None | None | Standard 7.10(lll) |
| CN-5 | The SSID of a wireless network shall not be broadcast. | 1 | None | None | Standard 7.5(kk) |
| CN-6 | Third party systems remotely accessing SCADA, Telemetry, and Control Systems shall conform to the ACME Information Security policy hardware guidelines. | 1 | None | None | Standard 7.6(ss) |
| CN-7 | Remote access shall be controlled by two-factor authentication controls. | 1 | None | None | Standard 7.6(pp) |
| CN-8 | Wireless networking traffic shall be encrypted encompassing WPA or WPA2 level security. | 1 | None | None | Standard 7.5(jj) |
| F-1 | Malicious firewall traffic shall be actively blocked | 1 | None | None | Standard 7.3(cc) |
| F-2 | MAC address filtering shall be applied to authorised wireless access points to disallow unauthorised network connections. | 1 | None | None | Standard 7.5(ll) |
| F-3 | The firewall shall disallow outbound SMB or WebDAV connections unless explicitly authorised. | 1 | Several past Windows Zero Day threats impact SCADA systems rely on outbound WebDav connections. | An unknown application attempts to open an SMB and WebDAV connection to an address that requires routing through the firewall and fails. | Anon |
| F-4 | Firewall administration activities shall be monitored. | 3 | Firewall is considered critical infrastructure. | Logs are changed when the firewall is updated, and the log indicates who made what change. | Anon |
| IC-2 | Pay numbers shall not be shared with unauthorised personnel. | 1 | None | None | Anon |
| IC-3 | Access credentials to SCADA, Telemetry, and Control Systems shall not be disclosed. | 1 | None | TBC | Standard 7.8(yy), Standard 7.9(iii) |
| ICTA-2 | All inbuilt security features shall be reviewed and, where appropriate, enabled. | 1 | None | None | Standard 7.10(nnn) |
| ICTA-3 | Product meta-data necessary to configure ACME security controls shall be provided before a product is installed in the Enterprise SCADA environment. | 1 | None | None | Standard 7.14(qqqq) |
| IDS-1 | ACME and ICT partner InfoSec teams shall be alerted to malicious firewall traffic. | 1 | None | None | Standard 7.3(dd), Standard 7.12(yyy) |

# Dependencies

## Day

Dependencies

| Depender | Dependee | Dependency | Type | Rationale |
| --- | --- | --- | --- | --- |
| Information Security Manager | Vendor | Product security accreditation | goal | ACME rely on vendors to provide proof of a product's level of security. |
| Information Security Manager | Vendor | Product security accreditation | goal | ACME rely on vendors to provide proof of a product's level of security. |
| Information Security Manager | Vendor | Product security accreditation | goal | ACME rely on vendors to provide proof of a product's level of security. |
| Information Security Manager | Vendor | Product security accreditation | goal | ACME rely on vendors to provide proof of a product's level of security. |
| Information Security Manager | Vendor | Product security accreditation | goal | ACME rely on vendors to provide proof of a product's level of security. |

## Night

Dependencies

| Depender | Dependee | Dependency | Type | Rationale |
| --- | --- | --- | --- | --- |
| Information Security Manager | Service Desk | Anomalous credentials reset | goal | Goal relies on service desk staff being switch-on enough to check logs, and pro-active enough to contact the ACME information security team. |